This is the Log File of everything done for the Project

Note: We would have to use Vitis AI regardless of the technique chosen for the compilation of the model. The VAI compiler is required to turn any model into “.xmodel” because that is the only type the FPGA UltraZed EV from Xilinx can run. Furthermore, it is required to install Vitis AI Runtime software in the petalinux image of the board so that inference can be done on the board. The steps of quantization by Vitis AI will be provided in Section 2.

Below is Section 1, the exact steps followed for building a petalinux project. For the sake of justifying,

why are we building a petalinux project? It is because the image created in this project is the one that we are supposed to flash in our FPGA board’s SD Card and it would comprise various tools, libraries, modules, softwares that we require for running a model.

# Section 1 Petalinux Project Build

Note: The prerequisites are the most important step of building this project because version compatibility of Ubuntu VM, Linux Kernel of the Ubuntu VM, Petalinux and Vitis AI is extremely important. If they do not match the tested list given on this page, then it can create a lot of errors and issues in the building which is something one does not wish to deal with.

The prerequisites of following the tutorial in this section are as follows:

- Download the Ubuntu 20.04 LTS. (from here) and create a VM in VMWare Workstation Pro. Make sure you keep enough disk space (500 GB preferred).

Note: The rest of the steps given below are to be executed inside the VM.

- Download petalinux installer “.run” version 2020.2 from the archive given on the AMD website. Note: You must have or create an account in AMD for this installation. Follow the steps given on this guide for the installation and make sure the document is of the correct version.

- Download the BSP zip file of UltraZed EV by accessing the SharePoint folder under section Reference Designs in the Avnet Page for the UltraZed EV SOM+CC FPGA Board (link).

Note: Make sure the BSP is 2020.2 only. Matching the version of the BSP and Petalinux installer is extremely important

- Unzip the BSP inside the VM. Check every component present inside every folder and then copy-paste the “.xsa” file and the zynqMP template folder outside of the unzipped BSP folder.

Finally, open the terminal and follow the below given steps and type each command exactly as written:

vm@ubuntu:~$ cd Desktop

vm@ubuntu:~/Desktop$ sudo vmhgfs-fuse .host:/Het /mnt/hgfs -o allow_other

Note: The second command is for enabling the shared folder which is named “Het” in the PC and the contents of which will reflect in /mnt/hgfs/ inside the VM. For this to work, shared folders has to be enabled in the VM’s settings beforehand.

vm@ubuntu:~/Desktop$ cd petal/

vm@ubuntu:~/Desktop/petal$ ls

components	settings.sh	doc	template-zynqMP	etc	tools petalinux_installation_log             uz7ev_evcc_base.xsa

petalinux-v2020.2-final-installer.run  uzev1

settings.csh

vm@ubuntu:~/Desktop/petal$ source settings.sh

PetaLinux environment set to '/home/vm/Desktop/petal'

WARNING: /bin/sh is not bash!

bash is PetaLinux recommended shell. Please set your default shell to bash.

Usage: grep [OPTION]... PATTERN [FILE]...

Try 'grep --help' for more information.

WARNING: This is not a supported OS

INFO: Checking free disk space

INFO: Checking installed tools

INFO: Checking installed development libraries

INFO: Checking network and other services

WARNING: No tftp server found - please refer to "UG1144 2020.2 PetaLinux Tools Documentation Reference Guide" for its impact and solution

Note: It is okay to ignore all mentioned warnings. The command “source settings.sh” is to be written inside the petalinux folder for every new session for the VM to identify the petalinux commands.

vm@ubuntu:~/Desktop/petal$ petalinux-create --type project --template zynqMP --name petalbuild

INFO: Create project: petalbuild

INFO: New project successfully created in /home/vm/Desktop/petal/petalbuild

vm@ubuntu:~/Desktop/petal$ cd petalbuild/

Note: Here is where you move the “.xsa” file and the zynqMP template folder that you has copy-pasted outside of the BSP folder. The “.xsa” file is the HDF required by Petalinux to create the project.

vm@ubuntu:~/Desktop/petal/petalbuild$ petalinux-config --get-hw-description .

INFO: Sourcing build tools

INFO: Getting hardware description...

INFO: Rename uz7ev_evcc_base.xsa to system.xsa

[INFO] Generating Kconfig for project

[INFO] Menuconfig project

*** End of the configuration.

*** Execute 'make' to start the build or try 'make help'.

[INFO] Extracting yocto SDK to components/yocto

[INFO] Sourcing build environment

[INFO] Generating kconfig for Rootfs

[INFO] Silentconfig rootfs

[INFO] Generating plnxtool conf

[INFO] Adding user layers

[INFO] Generating workspace directory

vm@ubuntu:~/Desktop/petal/petalbuild$ petalinux-config -c rootfs

Note: This opens the root configuration menu. Go to /misc/ and download all necessary packages you might require during the deployment. Python3 is a high preference as it will be required to run the test scripts for model evaluation.

INFO: Sourcing build tools

[INFO] Silentconfig project

[INFO] Generating kconfig for Rootfs

[INFO] Menuconfig rootfs

*** End of the configuration.

*** Execute 'make' to start the build or try 'make help'.

[INFO] Generating plnxtool conf

[INFO] Successfully configured rootfs

vm@ubuntu:~/Desktop/petal/petalbuild$ cd project-spec/meta-user/conf/

vm@ubuntu:~/Desktop/petal/petalbuild/project-spec/meta-user/conf$ nano petalinuxbsp.conf

Note: This is being done because I have tried building the image without this step and it has given the “do_cpio” error that is fixed by editing the “petalinuxbsp.conf”. The reference for this fix is from the original petalinux documentation here. Copy-paste the two lines given at this page inside the file and after saving and exiting, make sure to verify that file looks like what is given below (use cat –A).

vm@ubuntu:~/Desktop/petal/petalbuild/project-spec/meta-user/conf$ cat -A petalinuxbsp.conf

#User Configuration$

$

#OE_TERMINAL = "tmux"$

$

IMAGE_FSTYPES_remove = "cpio cpio.gz cpio.bz2 cpio.xz cpio.lzma cpio.lz4 cpio.gz.u-boot"$

IMAGE_FSTYPES_DEBUGFS_remove = "cpio cpio.gz cpio.bz2 cpio.xz cpio.lzma cpio.lz4 cpio.gz.u-boot"$

$

vm@ubuntu:~/Desktop/petal/petalbuild/project-spec/meta-user/conf$ cd ..

vm@ubuntu:~/Desktop/petal/petalbuild/project-spec/meta-user$ cd ..

vm@ubuntu:~/Desktop/petal/petalbuild/project-spec$ cd ..

vm@ubuntu:~/Desktop/petal/petalbuild$ petalinux-build

Note: this is the final step of building the project, depending on the number of cores and ram available, the process may take upto 5 hours or even more.

INFO: Sourcing build tools

[INFO] Building project

[INFO] Sourcing build environment

[INFO] Generating workspace directory

INFO: bitbake petalinux-image-minimal

Parsing recipes: 100% |#####################################################################################################################################################################################################################################################################################################################################################################| Time: 0:04:59

Parsing of 2995 .bb files complete (0 cached, 2995 parsed). 4265 targets, 172 skipped, 0 masked, 0 errors.

NOTE: Resolving any missing task queue dependencies

NOTE: Fetching uninative binary shim from file: ///home/vm/Desktop/petal/petalbuild/components/yocto/downloads/uninative/9498d8bba047499999a7310ac2576d0796461184965351a56f6d32c888a1f216/x86_64-nativesdk-libc.tar.xz;sha256sum=9498d8bba047499999a7310ac2576d0796461184965351a56f6d32c888a1f216

Initialising tasks: 100% |##################################################################################################################################################################################################################################################################################################################################################################| Time: 0:00:05

Checking sstate mirror object availability: 100% |##########################################################################################################################################################################################################################################################################################################################################| Time: 0:01:19

Sstate summary: Wanted 1661 Found 1358 Missed 303 Current 0 (81% match, 0% complete)

NOTE: Executing Tasks

NOTE: Setscene tasks completed

NOTE: Tasks Summary: Attempted 5663 tasks of which 4236 didn't need to be rerun and all succeeded.

INFO: Failed to copy built images to tftp dir: /tftpboot

[INFO] Successfully built project

vm@ubuntu:~/Desktop/petal/petalbuild$

The Petalinux project for our FPGA board has been built successfully. The project folder would be about 19 GBs or close. Check the folder contents to verify the download.

To create a bootable file which will go inside the SD card follow the commands:

Note: To run .xmodels which are quantized and compiled by Vitis AI, VART (Vitis AI Runtime has to be installed inside the petalinux-build and this can be done by copying the recipes folder of vitis ai inside the /project_name/project-spec/meta-user and building the project again.

There you have it, that is the end of Section 1. The next sections will be the step by step process of downloading Vitis AI and then converting the “.pt” model to the deployable “.xmodel” format using Vitis AI.

# Section 2 Vitis AI Setup

This section is the tutorial of downloading Docker to pull Vitis AI (version according to compatibility) and run the quantization and compilation process in Vitis AI. To verify once agin, these are the exact version we are using in this tutorial and they are compatible with each other.

| Ubuntu ISO | Linux Kernel | Petalinux | Vitis AI |
| --- | --- | --- | --- |
| 20.04 LTS | 5.10 | 2021.2 | 2.0 |

So, we will download docker first exactly as per the given instructions on its original page. (link)

Then we git clone Vitis AI and post that there are two options of installing Vitis AI. Set of commands to be followed from this link

- Pull the docker image and then run the .docker_run.sh file present inside the workspace folder

- Build a docker container using .docker_build.sh given inside the docker folder inside the workspace folder.

Note: The vitis ai tag used is the word “latest” when specifying version 2.0:

xilinx/vitis-ai-pytorch-cpu:2.0.0 (if building with option 1)

The run.sh file launches the workspace, post which you can activate the conda environment. We chose pytorch for this project as the original model has been built using the pytorch framework.

The docker can be launched to open the Vitis AI workspace by running this command in the Vitis AI folder in the terminal: sudo ./docker_run.sh xilinx/vitis-ai-pytorch-cpu:2.0.0

Then open this documentation for guide to download the vai_q_pytorch quantizer step by step.  Usually this is already instealled in vitis ai 2.0, but you have to follow this if using Vitis AI 3.0: (link)

Also, do not bother wavering outside the model list available in the Vitis AI repo because no matter how much they boast about having many models’, the support has NOT been added. So mnay of the issues in the github repo have been removed which prevents from reaching community discussions of errors, and even queries on the AMD platforms have sites which are now redundant. Vitis AI does not have support for transformer models yet and even deploying a simple model such as YOLO is hard enough. Custom operation is a tedious task to add in Vitis AI. Some of the below links are proof to this Claim that I m making here:

- The link provided to VCK5000 lounge by AMD here does not exist

- This issue 1252 withYolo has been removed, WHY!?

- Another issue 1411 from Vitis AI which had useful information. SMFH!!

And many more, which you can find if you take up a model which Vitis AI hasn’t provided ready made xmodels of.

Nevertheless I hate and ardently abhor the Vitis AI platform, in my opinion it tops the ryzenfall controversy. Will track the remaining flow of the project in the next section.

# Section 3 Yolov5s for Training and Quantizing with Vitis AI

This is a tutorial for importing, quantizing and compiling a model in Vitis AI 2.0. yes, surprise, we have moved to 2.0 right now because Vitis AI is a dumbass. Since DeepSC is an entirely custom made model, an autoencoder at that and ofcourse since vitis hasn’t added support yet, we will choose to train YOLO from scratch and quantize and train it using vitis 2.0.

### 3.1. Initial Setup and Model Training: The process begins by cloning the YOLOv5 repository. Next, you need to download a custom dataset fit for YOLOv5 from RoboFlow; the chosen project dataset is located at this link.

### 3.2. Model Modification and Execution: Before training, a crucial modification is required in the common.py file. Change the default activation function from SiLU to LeakyReLU(0.1) by setting: default_act = LeakyReLU(0.1)

After this change, train the model using train.py, specifying an image size of 640 (img 640), and ensure the trained model is saved. Follow the specified notebook for detailed training guidance.

### 3.3. Workspace and Calibration Preparation: Open the yolov5 folder within the Vitis AI workspace (LAUNCH THE DOCKER) and place the dataset inside it. The directory structure after training should show folders like benchmarks.py, data, models, and your dataset folder, e.g., chair-3. Inside, you should find data.yaml, train, valid, etc.

- Install additional libraries inside the yolov5 folder: pip install ultralytics and pip install requirements.txt.

- Create a calibration dataset list for quantization: find chair-3/train/images -name "*.jpg" | head -n 100 > calib_list.txt.

- Install a specific version of setuptools, which is important for the quant code to run correctly: pip install setuptools-58.0.4.

### 3.4. Quantization Code Modification: Open the yolo.py file and replace the forward method in the Detect class with the following snippet. This change is necessary because returning a list causes an error, and returning a tuple resolves it:

The method should be:

def forward(self, x):

for i in range(self.nl):

x[i] = self.m[i](x[i])  # conv

return tuple(x) # This returns a tuple

Finally, create a file called post_process.py inside the yolov5 folder. Add the following code inside it:

### 3.5. Model Quantization and Compilation

This section handles the quantization of the trained model and its compilation for the target hardware.

- Calibration: Run the quantization script in calibration mode to determine the fixed-point parameters

- Test/Quantization: Run the quantization script in test mode to generate the fixed-point .xmodel

## Save this file and then create a quantization code in a new file quantize.py (Expand to see).

### import os

### import sys

### import argparse

### import torch

### from torch.utils.data import Dataset

### import torchvision

### from torchvision.io import read_image

### from pytorch_nndct.apis import torch_quantizer

### from models.common import DetectMultiBackend

### class CustomImageDataset(Dataset):

### def __init__(self, label_dir, img_dir, width, height):

### self.label_dir = label_dir

### self.img_dir = img_dir

### self.width = width

### self.height = height

### self.img_names = [os.path.splitext(f)[0] for f in os.listdir(img_dir)]

### def __len__(self):

### return len(self.img_names)

### def __getitem__(self, idx):

### img_name = self.img_names[idx]

### img_path = os.path.join(self.img_dir, img_name + ".jpg")

### label_path = os.path.join(self.label_dir, img_name + ".txt")

### image = read_image(img_path).float() / 255.0

### image = torchvision.transforms.Resize((self.width, self.height))(image)

### boxes, labels = [], []

### with open(label_path) as f:

### for line in f.readlines():

### vals = line.strip().split()

### labels.append(int(vals[0]))

### x0 = (float(vals[1]) - float(vals[3]) / 2) * self.width

### y0 = (float(vals[2]) - float(vals[4]) / 2) * self.height

### x1 = (float(vals[1]) + float(vals[3]) / 2) * self.width

### y1 = (float(vals[2]) + float(vals[4]) / 2) * self.height

### boxes.append([x0, y0, x1, y1])

### target = {

### "boxes": torch.tensor(boxes, dtype=torch.float32),

### "labels": torch.tensor(labels, dtype=torch.int64),

### "image_id": torch.tensor([idx + 1])

### }

### return image, target

### def quantize(build_dir, quant_mode, weights, dataset_dir):

### quant_model_dir = os.path.join(build_dir, 'quant_model')

### device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

### # Load model

### model = DetectMultiBackend(weights=weights).to(device)

### rand_input = torch.randn(1, 3, 640, 640).to(device)

### # Create quantizer

### quantizer = torch_quantizer(quant_mode, model, rand_input, output_dir=quant_model_dir)

### quantized_model = quantizer.quant_model.to(device)

### # Dataset for calibration / test

### dataset = CustomImageDataset(

### os.path.join(dataset_dir, 'labels'),

### os.path.join(dataset_dir, 'images'),

### width=640, height=640

### )

### loader = torch.utils.data.DataLoader(dataset, batch_size=1, shuffle=False)

### quantized_model.eval()

### with torch.no_grad():

### for img, target in loader:

### _ = quantized_model(img.to(device))  # Run raw outputs only

### if quant_mode == 'calib':

### quantizer.export_quant_config()

### elif quant_mode == 'test':

### quantizer.export_xmodel(deploy_check=False, output_dir=quant_model_dir)

### def main():

### parser = argparse.ArgumentParser()

### parser.add_argument('-b', '--build_dir', type=str, default='build')

### parser.add_argument('-q', '--quant_mode', type=str, default='calib', choices=['calib','test'])

### parser.add_argument('-w', '--weights', type=str, required=True)

### parser.add_argument('-d', '--dataset', type=str, required=True)

### args = parser.parse_args()

### quantize(args.build_dir, args.quant_mode, args.weights, args.dataset)

### if __name__ == '__main__':

### main()

### 3.5. Model Quantization and Compilation: This section handles the quantization of the trained model and its compilation for the target hardware.

Calibration: Run the quantization script in calibration mode to determine the fixed-point parameters: Run this file using vitis ai by this command: python quantize.py --quant_mode calib

Test/Quantization: Run the quantization script in test mode to generate the fixed-point .xmodel: After this run the command: python quantize.py --quant_mode test

Compilation: Compile the quantized model for the ZCU104 chip using the Vitis AI compiler: Run this command for compilation for the zcu104 chip:

vai_c_xir --xmodel ./quantize_result/DetectionModel_int.xmodel --arch /opt/vitis_ai/compiler/arch/DPUCZDX8G/ZCU104/arch.json --output_dir ./compiled_model/ --net_name yolov5s_zcu104

Will give output:

**************************************************

* VITIS_AI Compilation - Xilinx Inc.

**************************************************

[UNILOG][INFO] Compile mode: dpu

[UNILOG][INFO] Debug mode: null

[UNILOG][INFO] Target architecture: DPUCZDX8G_ISA1_B4096

[UNILOG][INFO] Graph name: DetectionModel, with op num: 448

[UNILOG][INFO] Begin to compile...

[UNILOG][INFO] Total device subgraph number 5, DPU subgraph number 1

[UNILOG][INFO] Compile done.

[UNILOG][INFO] The meta json is saved to "/workspace/yolov5/./compiled_model/meta.json"

[UNILOG][INFO] The compiled xmodel is saved to "/workspace/yolov5/./compiled_model//yolov5s_zcu104.xmodel"

[UNILOG][INFO] The compiled xmodel's md5sum is b5acfbeea0e0e865480461812536bd42, and has been saved to "/workspace/yolov5/./compiled_model/md5sum.txt"

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5$ cd quantize_result/

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5/quantize_result$ ls

bias_corr.pth  deploy_check_data_int  DetectionModel_int.xmodel  DetectionModel.py  __pycache__  quant_info.json

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5/quantize_result$ cd ..

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5$ cd compiled_model/

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5/compiled_model$ ls

md5sum.txt  meta.json  yolov5s_zcu104.xmodel

(vitis-ai-pytorch) vitis-ai-user@vm:/workspace/yolov5/compiled_model$

The “.xmodel” inside the compile folder is ready to be tested on the FPGA.

## After writing and saving these files, run the quant.py with modes calib and test one by one.

## Output of “calib” mode and “test” mode for quantize.py (Expand to see)

(vitis-ai-pytorch) Vitis-AI /workspace/yolov5_fire > python quantize.py -q calib -w best.pt -d calib_dataset

/opt/vitis_ai/conda/envs/vitis-ai-pytorch/lib/python3.6/site-packages/torch/cuda/__init__.py:52: UserWarning: CUDA initialization: Found no NVIDIA driver on your system. Please check that you have an NVIDIA GPU and installed a driver from http://www.nvidia.com/Download/index.aspx (Triggered internally at  /opt/conda/conda-bld/pytorch_1607370120218/work/c10/cuda/CUDAFunctions.cpp:100.)

return torch._C._cuda_getDeviceCount() > 0

No CUDA runtime is found, using CUDA_HOME='/usr/local/cuda'

[VAIQ_NOTE]: Loading NNDCT kernels...

[INFO] Model loaded successfully from ['best.pt']

[VAIQ_WARN]: CUDA is not available, change device to CPU

[VAIQ_NOTE]: Quantization calibration process start up...

[VAIQ_NOTE]: =>Quant Module is in 'cpu'.

[VAIQ_NOTE]: =>Parsing DetectMultiBackend...

[VAIQ_NOTE]: Start to trace model...

[VAIQ_NOTE]: Finish tracing.

[VAIQ_NOTE]: Processing ops...

██████████████████████████████████████████████████| 218/218 [00:00<00:00, 2378.39it/s, OpInfo: name = DetectMultiBackend/DetectionModel[model]/Detect[model]/Detect[2

[VAIQ_NOTE]: =>Doing weights equalization...

[VAIQ_NOTE]: =>Quantizable module is generated.(build/quant_model/DetectMultiBackend.py)

[VAIQ_NOTE]: =>Get module with quantization.

[VAIQ_NOTE]: =>Exporting quant config.(build/quant_model/quant_info.json)

(vitis-ai-pytorch) Vitis-AI /workspace/yolov5_fire > python quantize.py -q test -w best.pt -d calib_dataset

/opt/vitis_ai/conda/envs/vitis-ai-pytorch/lib/python3.6/site-packages/torch/cuda/__init__.py:52: UserWarning: CUDA initialization: Found no NVIDIA driver on your system. Please check that you have an NVIDIA GPU and installed a driver from http://www.nvidia.com/Download/index.aspx (Triggered internally at  /opt/conda/conda-bld/pytorch_1607370120218/work/c10/cuda/CUDAFunctions.cpp:100.)

return torch._C._cuda_getDeviceCount() > 0

No CUDA runtime is found, using CUDA_HOME='/usr/local/cuda'

[VAIQ_NOTE]: Loading NNDCT kernels...

[INFO] Model loaded successfully from ['best.pt']

[VAIQ_WARN]: CUDA is not available, change device to CPU

[VAIQ_NOTE]: Quantization test process start up...

[VAIQ_NOTE]: =>Quant Module is in 'cpu'.

[VAIQ_NOTE]: =>Parsing DetectMultiBackend...

[VAIQ_NOTE]: Start to trace model...

[VAIQ_NOTE]: Finish tracing.

[VAIQ_NOTE]: Processing ops...

██████████████████████████████████████████████████| 218/218 [00:00<00:00, 3003.59it/s, OpInfo: name = DetectMultiBackend/DetectionModel[model]/Detect[model]/Detect[2

[VAIQ_NOTE]: =>Doing weights equalization...

[VAIQ_NOTE]: =>Quantizable module is generated.(build/quant_model/DetectMultiBackend.py)

[VAIQ_NOTE]: =>Get module with quantization.

[VAIQ_NOTE]: =>Converting to xmodel ...

[VAIQ_NOTE]: =>Successfully convert 'DetectMultiBackend' to xmodel.(build/quant_model/DetectMultiBackend_int.xmodel)

## Compilation

Compile the quantized model for the ZCU104 chip using the Vitis AI compiler: Run this command for compilation for the zcu104 chip:

vai_c_xir --xmodel ./quantize_result/DetectionModel_int.xmodel --arch /opt/vitis_ai/compiler/arch/DPUCZDX8G/ZCU104/arch.json --output_dir ./compiled_model/ --net_name yolov5s_zcu104

## Output of Compilation (Expand to see)

(vitis-ai-pytorch) Vitis-AI /workspace/yolov5_fire > vai_c_xir     -x build/quant_model/newmodel/DetectMultiBackend_int.xmodel     -a /opt/vitis_ai/compiler/arch/DPUCZDX8G/ZCU104/arch.json     -o compiled_model     -n dpu_yolov5

**************************************************

* VITIS_AI Compilation - Xilinx Inc.

**************************************************

[UNILOG][INFO] Compile mode: dpu

[UNILOG][INFO] Debug mode: function

[UNILOG][INFO] Target architecture: DPUCZDX8G_ISA0_B4096_MAX_BG2

[UNILOG][INFO] Graph name: DetectMultiBackend, with op num: 475

[UNILOG][INFO] Begin to compile...

[UNILOG][INFO] Total device subgraph number 5, DPU subgraph number 1

[UNILOG][INFO] Compile done.

[UNILOG][INFO] The meta json is saved to "/workspace/yolov5_fire/compiled_model/meta.json"

[UNILOG][INFO] The compiled xmodel is saved to "/workspace/yolov5_fire/compiled_model/dpu_yolov5.xmodel"

[UNILOG][INFO] The compiled xmodel's md5sum is c0870e414d1751855dda85a54051c03d, and has been saved to "/workspace/yolov5_fire/compiled_model/md5sum.txt"

(vitis-ai-pytorch) Vitis-AI /workspace/yolov5_fire >

# Section 4 Transfer files needed to be run to FPGA

This section is to guide user on how to boot the FPGA board when it is already flashed with the petalinux image.   This is required because files need to be transferred from the local machine into the FPGA board to run them.

Use Putty on an Ubuntu VM to open the FPGA terminal using serial connection  (/dev/ttyUSB1). Switch the board on and off if you don’t see boot automatically.

Note: USB0 is not working and CP210x Driver is required if using Windows

First assign IP address to “eth0” interface using command: sudo ip addr add 192.168.55.2/24 dev eth0

Then set the interface up using this command: sudo ip link set eth0 up

Then open a new terminal and open the access to FPGA board using the command:             ssh root@192.168.55.2  and enter the passowrd (root).

To copy content from the Ubuntu machine to the board:                                                       scp ./path/to/file_or_folder/ root@<FPGA_ETH0_IP>:/home/root/path_to_directory_in_board/

# Section 5 Running Inference on FPGA

This section is the guide of how to use VART to run inference of a model on the FPGA board i.e. running inference of the above quatized Yolo Model on the FPGA board.

Copy a folder from the VM to the board using section 4’s instructions. The folder MUST have the following contents:

- bias_corr.pth

- md5sum.txt

- quant_info.json

- DetectMultiBackend_int.xmodel

- meta.json

- inference.py

- DetectMultiBackend.py

- test.jpg

- dpu_yolov5.xmodel (the main compiled model)

Except inference.py and test.jpg all other files have been created by Vitis AI during quantization and compilation.

# Note

For all of this the Python files to be used are given inside the project, so please go through them and use them. If anything does not work for you, then you didn’t follow up well. Because after 1000000000000000000 errors I finally made it work and created this log file. You cannot begin to imagine the hate I have for Vitis AI so running this “simple” object detection model on the board with no one else to help was a huge thing for me.